ENTERPRISE NETWORK DESIGN
AND IMPLEMENTATION

Cisco Packet Tracer Practical Lab Report

Topics: VLANs • Trunking • Inter-VLAN Routing • STP • WLAN • Servers • Troubleshooting • Redundancy

Name:Eliud Kigen Kipkorir
                                                                               Date:10-10-2026

# Document purpose and scope

This report documents the enterprise network built and tested in Cisco Packet Tracer. It explains the design from the initial topology through VLAN creation, trunk configuration, router-on-a-stick inter-VLAN routing, end-device addressing, wireless access, server testing, Spanning Tree Protocol (STP), and redundant-link failover. Commands are grouped by the device on which they are entered.

Note on accuracy: this report records the configuration and observed results from the lab conversation. Some command blocks are consolidated configuration sequences that reproduce the described final configuration; they are not a verbatim transcript of every keystroke.

# 1. Objectives

Design a small enterprise network using a router, three Layer 2 switches, wired end devices, a wireless access point, a laptop, and a server.

Separate departments and services using VLANs and IPv4 subnets.

Configure access ports for end devices and 802.1Q trunk links between switches and the router.

Enable communication between VLANs using router-on-a-stick.

Configure a wireless network and test wireless-to-wired connectivity.

Use STP to prevent Layer 2 loops while preserving a redundant path.

Demonstrate failover and recovery when a trunk link is disconnected and restored.

Verify the design with show commands, ping tests, and an HTTP browser test.

# 2. Equipment and topology

| Device | Quantity | Purpose |
| --- | --- | --- |
| Cisco 2911 Router (R0) | 1 | Default gateways and routing between VLANs |
| Cisco 2960 Switches (SW0, SW1, SW2) | 3 | Layer 2 switching, VLANs, trunks, and STP |
| Engineering PC (ENG) | 1 | VLAN 10 end-device test |
| Finance PC (FIN) | 1 | VLAN 20 end-device test |
| Management PC (MGMT) | 1 | VLAN 30 end-device test |
| Server PC | 1 | Additional host in VLAN 40 |
| Dedicated Server | 1 | Server at 192.168.40.20; HTTP test target |
| Access Point (AP0) | 1 | Wireless access for laptop |
| Laptop | 1 | Wireless client in VLAN 30 |

## 2.1 Physical connections

| From device / port | To device / port | Purpose |
| --- | --- | --- |
| R0 G0/0 | SW0 Fa0/1 | 802.1Q trunk to router-on-a-stick |
| SW0 Fa0/2 | SW1 Fa0/1 | Switch trunk; one side of redundant triangle |
| SW0 Fa0/3 | SW2 Fa0/1 | Switch trunk; second side of triangle |
| SW1 Fa0/2 | ENG FastEthernet0 | Engineering access port |
| SW1 Fa0/3 | FIN FastEthernet0 | Finance access port |
| SW1 Fa0/4 | SW2 Fa0/6 | Additional redundant trunk link |
| SW2 Fa0/2 | MGMT FastEthernet0 | Management access port |
| SW2 Fa0/3 | Server PC FastEthernet0 | Server PC access port |
| SW2 Fa0/4 | Dedicated Server FastEthernet0 | Server access port |
| SW2 Fa0/5 | AP0 Ethernet port | Wireless AP uplink in VLAN 30 |
| Laptop wireless | AP0 SSID Lude-Lab | Wireless connection |

Use the appropriate copper Ethernet cable or Packet Tracer's automatic connection tool. The exact physical port should always be confirmed from the labels shown on the links in the saved topology.

# 3. IP addressing and VLAN plan

| VLAN | Name | Network / mask | Default gateway | Purpose |
| --- | --- | --- | --- | --- |
| 10 | ENGINEERING | 192.168.10.0 /24 | 192.168.10.1 | Engineering department |
| 20 | FINANCE | 192.168.20.0 /24 | 192.168.20.1 | Finance department |
| 30 | MANAGEMENT | 192.168.30.0 /24 | 192.168.30.1 | Management and wireless client |
| 40 | SERVERS | 192.168.40.0 /24 | 192.168.40.1 | Servers and server PC |

| Host | IPv4 address | Subnet mask | Default gateway |
| --- | --- | --- | --- |
| ENG PC | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| FIN PC | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |
| MGMT PC | 192.168.30.10 | 255.255.255.0 | 192.168.30.1 |
| Laptop (Wi-Fi) | 192.168.30.20 | 255.255.255.0 | 192.168.30.1 |
| Server PC | 192.168.40.10 | 255.255.255.0 | 192.168.40.1 |
| Dedicated Server | 192.168.40.20 | 255.255.255.0 | 192.168.40.1 |

A /24 network uses mask 255.255.255.0. Each VLAN is a separate Layer 2 broadcast domain and has its own IP subnet. Hosts in different VLANs need a Layer 3 device to communicate; in this lab, the router provides that service.

# 4. Build the topology and save the file

Open Cisco Packet Tracer and place one Cisco 2911 router, three Cisco 2960 switches, the four PCs, the dedicated Server, an Access Point, and a laptop.

Arrange SW0 near the router and SW1/SW2 below it. Connect SW0–SW1 and SW0–SW2. Add the SW1–SW2 link to form a triangle.

Connect the wired end devices to their assigned switch ports using the physical connection table in Section 2.1.

Connect the router to SW0 on G0/0 to Fa0/1.

Associate the laptop with the wireless access point after configuring its SSID and security.

Save the Packet Tracer file as Lude_Enterprise_Network_Design_and_Implementation.pkt. Keep a backup copy when the project is complete.

# 5. Create VLANs on all switches

Create VLANs 10, 20, 30, and 40 on SW0, SW1, and SW2. The VLAN database must contain the same VLAN IDs on all switches that carry those VLANs.

enable

configure terminal

vlan 10

name ENGINEERING

exit

vlan 20

name FINANCE

exit

vlan 30

name MANAGEMENT

exit

vlan 40

name SERVERS

exit

end

show vlan brief

Explanation:

enable enters privileged EXEC mode, where verification and configuration commands are available.

configure terminal enters global configuration mode.

vlan <ID> creates or selects a VLAN; name gives it a readable label.

exit returns one configuration level; end returns to privileged EXEC mode.

show vlan brief verifies that the VLANs exist and shows access-port membership.

Repeat this VLAN creation sequence on SW0, SW1, and SW2.

# 6. Configure access ports

An access port normally carries traffic for one data VLAN and is used for an end device such as a PC, server, or access point. The following are the final intended access-port assignments.

## 6.1 SW1 — Engineering and Finance

enable

configure terminal

interface fastEthernet 0/2

switchport mode access

switchport access vlan 10

exit

interface fastEthernet 0/3

switchport mode access

switchport access vlan 20

exit

end

show vlan brief

show interfaces fa0/2 switchport

show interfaces fa0/3 switchport

## 6.2 SW2 — Management, servers, and access point

enable

configure terminal

interface fastEthernet 0/2

switchport mode access

switchport access vlan 30

exit

interface fastEthernet 0/3

switchport mode access

switchport access vlan 40

exit

interface fastEthernet 0/4

switchport mode access

switchport access vlan 40

exit

interface fastEthernet 0/5

switchport mode access

switchport access vlan 30

exit

end

show vlan brief

show interfaces fa0/2 switchport

show interfaces fa0/3 switchport

show interfaces fa0/4 switchport

show interfaces fa0/5 switchport

Fa0/2 is for the MGMT PC; Fa0/3 is for the Server PC; Fa0/4 is for the dedicated Server; Fa0/5 is for AP0. If the actual cabling differs in the saved file, adjust the interface commands to match the connected devices.

# 7. Configure 802.1Q trunk ports

A trunk carries traffic for multiple VLANs between network devices. This lab uses trunks between SW0 and both downstream switches, plus the redundant SW1–SW2 trunk. SW0 Fa0/1 is also configured as a trunk toward the router for router-on-a-stick.

## 7.1 SW0 trunk ports

enable

configure terminal

interface fastEthernet 0/1

switchport mode trunk

exit

interface fastEthernet 0/2

switchport mode trunk

exit

interface fastEthernet 0/3

switchport mode trunk

exit

end

show interfaces trunk

## 7.2 SW1 trunk ports

enable

configure terminal

interface fastEthernet 0/1

switchport mode trunk

exit

interface fastEthernet 0/4

switchport mode trunk

exit

end

show interfaces trunk

show interfaces fa0/4 switchport

## 7.3 SW2 trunk ports

enable

configure terminal

interface fastEthernet 0/1

switchport mode trunk

exit

interface fastEthernet 0/6

switchport mode trunk

exit

end

show interfaces trunk

show interfaces fa0/6 switchport

The observed output showed the trunk encapsulation as 802.1Q, status trunking, and VLANs 1, 10, 20, 30, and 40 active and allowed. On 2960 switches, 802.1Q is the normal trunk encapsulation; the exact available commands can vary slightly by Packet Tracer model.

# 8. Configure router-on-a-stick

Router-on-a-stick uses one physical router interface and multiple logical subinterfaces. Each subinterface is tagged for one VLAN using 802.1Q and receives the default-gateway IP address for that VLAN.

enable

configure terminal

interface gigabitEthernet 0/0

no shutdown

exit

interface gigabitEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

interface gigabitEthernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

interface gigabitEthernet 0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

exit

interface gigabitEthernet 0/0.40

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

exit

end

show ip interface brief

show ip route

Explanation:

interface gigabitEthernet 0/0 selects the physical router port connected to SW0.

no shutdown enables the physical interface. The connected router and switch ports must be cabled and active.

G0/0.10, G0/0.20, G0/0.30, and G0/0.40 are logical subinterfaces associated with VLAN IDs 10, 20, 30, and 40.

encapsulation dot1Q <VLAN-ID> tells the router which tagged VLAN to handle.

ip address assigns the default gateway for that subnet.

show ip interface brief should show the physical interface and all four subinterfaces as up/up when the link and VLAN configuration are correct.

show ip route should show connected routes for the four directly connected subnets.

# 9. Configure end-device IP addresses

For each PC or server in Packet Tracer, click the device, choose Desktop → IP Configuration, select Static, and enter the IP address, subnet mask, and default gateway from Section 3. Use the gateway matching the device's VLAN.

The laptop is also assigned a static address after connecting to the wireless network: 192.168.30.20 /24 with gateway 192.168.30.1. Do not assign the same IP address to two devices.

# 10. Configure the wireless LAN

Click AP0 and open its wireless configuration interface (the exact tab can vary by Packet Tracer access-point model).

Set the SSID to Lude-Lab.

Select WPA2-PSK security if supported by the model and enter the pre-shared key Lude@12345.

Ensure the AP Ethernet port is connected to SW2 Fa0/5, configured as an access port in VLAN 30.

On the laptop, open Desktop → PC Wireless (or the wireless utility available for that laptop model), select Lude-Lab, enter the key, and connect.

Open Desktop → IP Configuration on the laptop and set IP 192.168.30.20, mask 255.255.255.0, and gateway 192.168.30.1.

The access point bridges the wireless client into the wired VLAN. A separate AP management address such as 192.168.30.2 was considered but was not required for the successful client traffic tests in this lab.

# 11. Configure and test the server

The dedicated Server is assigned 192.168.40.20/24 with gateway 192.168.40.1. In Packet Tracer, open the Server, select Services, and check that HTTP is enabled. The browser test confirmed that the default Packet Tracer welcome page loaded.

Click Server → Desktop → IP Configuration and confirm 192.168.40.20, 255.255.255.0, and 192.168.40.1.

Click Server → Services → HTTP and ensure the HTTP service is On (if this option exists in the model).

From the laptop, open Desktop → Web Browser and enter http://192.168.40.20.

The default Cisco Packet Tracer welcome page loaded successfully, confirming application-layer HTTP access to the server.

# 12. Configure Spanning Tree Protocol and redundant links

The three-switch triangle gives the network a redundant Layer 2 path. Without loop prevention, Ethernet frames can circulate around a switching loop. STP calculates a loop-free active topology and places a redundant port into a blocking/alternate state for a VLAN.

## 12.1 Check STP on SW1, SW2, and SW0

enable

show spanning-tree vlan 10

This command was run on the switches to inspect the root bridge, root port, designated ports, alternate ports, and port states for VLAN 10. The switch with the lowest Bridge ID becomes root; the output identified root bridge address 0000.0CBD.1973.

## 12.2 Check trunks

show interfaces trunk

This confirms which interfaces are trunking, which VLANs are allowed and active, and which VLANs are currently forwarding and not pruned on each trunk.

## 12.3 Observed normal state on SW0

| Interface | Role | State | Interpretation |
| --- | --- | --- | --- |
| Fa0/1 | Designated | Forwarding | Router-facing trunk |
| Fa0/2 | Root | Forwarding | Preferred path toward the root bridge |
| Fa0/3 | Alternate | Blocking | Redundant path held back to prevent a loop |

Observed SW0 output showed Fa0/2 as Root FWD and Fa0/3 as Altn BLK. SW0 was not the root bridge; it had a root port pointing toward the root bridge.

## 12.4 Failover test

With the triangle connected, check show spanning-tree vlan 10 on SW0. Fa0/2 was the root port and Fa0/3 was alternate/blocking.

Temporarily disconnect the cable between SW0 Fa0/2 and SW1 Fa0/1.

Wait for STP to reconverge, then run show spanning-tree vlan 10 on SW0 again.

The observed output showed Fa0/3 becoming Root FWD, with the root port listed as FastEthernet0/3. Traffic could now use the path through SW2.

Reconnect the SW0 Fa0/2–SW1 Fa0/1 cable and allow STP to reconverge.

The final observed output returned to Fa0/2 Root FWD and Fa0/3 Altn BLK.

This is a successful failover and recovery demonstration: the alternate path became active when the preferred path failed, then STP returned to the normal loop-free topology when the link was restored.

# 13. Verification commands used

| Command | Purpose |
| --- | --- |
| show vlan brief | Lists VLANs and access-port assignments. |
| show interfaces trunk | Displays trunk ports, VLANs allowed/active, and VLAN forwarding state. |
| show interfaces fa0/x switchport | Shows switchport mode, trunk/access status, and VLAN settings for a specific port. |
| show spanning-tree vlan 10 | Displays STP root information, port roles, and states for VLAN 10. |
| show ip interface brief | Checks router interface and subinterface IP addresses and operational states. |
| show ip route | Displays the router's routing table. |
| ping <IP-address> | Tests IP reachability between two devices. |
| exit | Leaves the current configuration level or CLI session. |
| end | Returns from configuration mode to privileged EXEC mode. |

# 14. Connectivity and application tests

| Test | Observed result | Conclusion |
| --- | --- | --- |
| ENG PC → FIN PC (192.168.20.10) | Second ping: 4/4 replies, 0% loss | Inter-VLAN routing worked. |
| ENG PC → MGMT PC (192.168.30.10) | 4/4 replies, 0% loss | Communication between VLANs worked. |
| ENG PC → Server (192.168.40.20) | 4/4 replies, 0% loss; later 4/4, 0% loss, avg 21 ms | End-to-end routing to server VLAN worked. |
| Server PC → Server (192.168.40.20) | 4/4 replies, 0% loss | Hosts in VLAN 40 communicated. |
| Laptop → gateway (192.168.30.1) | 4/4 replies, 0% loss | Wireless client reached its gateway. |
| Laptop → Server (192.168.40.20) | 4/4 replies, 0% loss | Wireless-to-wired and inter-VLAN connectivity worked. |
| Laptop browser → http://192.168.40.20 | Default Packet Tracer welcome page loaded | HTTP service was reachable. |
| STP failover on SW0 | Fa0/3 changed from Altn BLK to Root FWD when SW0–SW1 link was disconnected | Redundant path took over. |
| STP recovery on SW0 | Fa0/2 returned to Root FWD and Fa0/3 returned to Altn BLK after reconnection | STP restored the normal loop-free topology. |

A first ping can occasionally time out or take longer while ARP resolves a destination MAC address or the simulation initializes. Repeat the ping before diagnosing a persistent failure.

# 15. Troubleshooting method

The lab used a structured troubleshooting sequence:

Observe the symptom: identify which device or path is failing.

Identify the likely layer: physical link, VLAN/access port, trunk, IP addressing, gateway, routing, STP, or service.

Test with focused commands such as show vlan brief, show interfaces trunk, show spanning-tree vlan 10, show ip interface brief, and ping.

Correct one configuration item at a time.

Repeat the test to verify the fix.

## Common problems and checks

| Symptom | Likely checks |
| --- | --- |
| Ping to another VLAN fails | Check host IP/mask/gateway, access VLAN, trunk VLAN list, router subinterface and encapsulation. |
| Trunk does not appear | Check cable/link, interface status, and switchport mode trunk on both ends. |
| VLAN not forwarding on trunk | Check that the VLAN exists, is active, and STP is not blocking the path. |
| Wireless client cannot connect | Check SSID, security mode/key, AP uplink, VLAN 30 access port, and laptop IP/gateway. |
| Server ping works but web page fails | Check server IP/gateway and HTTP service status. |
| Unexpected CLI error after typing C:\\> | C:\\> is the Windows PC prompt, not a Cisco switch command; use exit to leave the switch CLI. |

# 16. Results and discussion

The project achieved the main design objectives. VLANs separated Engineering, Finance, Management/WLAN, and Servers into different broadcast domains. Router-on-a-stick enabled communication between those VLANs. The laptop joined the Lude-Lab wireless network and reached the wired server. The HTTP test confirmed that the server's web service was accessible. Finally, STP prevented the triangle topology from forwarding on every path simultaneously and demonstrated recovery when a trunk link failed.

The ping results showed successful end-to-end communication with 0% packet loss in the recorded tests. The final STP output on SW0 confirmed Fa0/2 as the root port in normal operation and Fa0/3 as the alternate blocking port. During the failure test, Fa0/3 became the root forwarding port, and after the cable was restored it returned to alternate blocking.

# 17. Conclusion

The enterprise network was successfully designed, configured, and verified in Cisco Packet Tracer. The lab combined Layer 2 switching concepts (VLANs, access ports, trunks, and STP) with Layer 3 routing (router-on-a-stick), wireless access, server services, and practical troubleshooting. The redundant-link experiment showed how STP prevents switching loops while preserving an alternate path for recovery. The completed project provides a practical foundation for more advanced routing, network security, and enterprise network design.

# 18. Screen shot